KINGFISHER III

RIGGING AND MAINTENANCE INSTRUCTIONS

American Fiberglass Corp.

Post Office Box 9155

Hanahan, South Carolina

29410

KINGFISHER  III RIGGING INSTRUCTIONS

RIGGING THE MAST (REFER TO THE DRAWING)

ATTACH THE JIB HALYARD BLOCK TO THE LOWER TANG WITH THE SHACKLE PROVIDED.

ATTACH THE JIB STAY TO THE UPPER TANG WITH THE CLEVIS AND COTTER PIN, INSERT THE CLEVIS SO THE COTTER PIN IS ON THE MAST SIDE OF THE TANG.

ATTACH THE PORT AND STARBOARD SHROUDS TO THE SHROUD TANGS. THE SHROUDS ARE THE 2 LONGER STAINLESS WIRES

(THE JIB STAY IS ABOUT 4” SHORTER)

THE FORKED END OF THE SHROUD AND THE STAY WIRES IS ATTACHED TO THE MAST TANGS, THE EYE END WILL BE ATTACHED IN THE ADJUSTER.

RUN THE JIB HALYARD ROPE (23') THROUGHT THE JIB HALYARD BLOCK AND SECURE IT TO THE HALYARD CLEAT ON THE MAST.

RUN THE MAIN HALYARD ROPE (30') THROUGH THE MAST HEAD SHEAVE AND SECURE TO THE OTHER HALYARD CLEAT ON THE MAST.

THE MAST IS NOW READY FOR STEPPING. CARE MUST BE TAKEN SO AS NOT TO SCRATCH THE FRONT SEAT WHEN STEPPING THE MAST. MAKE SURE THAT THE MAST IS SEATED ON THE MAST STEP BASE CASTING. A SLIGHT TWISTING HELPS. THE MAST WILL STAND WITHOUT AID OF THE SHROUDS AND JIB STAY.

- 1 -

RIGGING THE HULL

ATTACH THE ADJUSTERS TO THE  3  DECK CHAIN PLATES. FACE THE ADJUSTERS SO THE OPEN SIDE FACES AFT.THE ADJUSTERS ARE ATTACHED WITH THE CLEVIS PINS PROVIDED AND A COTTER PIN SECURES THE CLEVIS. REMOVE THE UPPER CLEVIS PINS FROM THE ADJUSTERS AND HOLD.

ATTACH THE  JIB STAY TO THE FORWARD  ADJUSTER IN A MANNER SO THAT THE MAST IS ABOUT IN THE MIDDLE OF THE HOLE IN THE FRONT SEAT WHEN A SLIGHT BACK PRESSURE IS APPLIED.

ATTACH THE SHROUDS TO THE ADJUSTERS SO THAT THE CLEVIS PINS ARE AN EQUAL NUMBER OF HOLES FROM THE TOP  OF THE ADJUSTER ON EACH SIDE. THE SHROUD SHOULD BE AS TIGHT AS POSSIBLE AND THE MAST SHOULD NOT LEAN TO EITHER SIDE.

RIGGING THE BOOM

ATTACH THE SWIVEL BLOCK ON THE SLIDE TO THE SMALL TRACK ON THE BOTTOM OF THE BOOM AND TIGHTEN THE KNRULED SET SCREW, THIS BLOCK IS REMOVED WHEN USING THE ROLLER REEFING FEATURE.

ATTACH THE BECKET BLOCK TO THE SWIVEL TANG ON THE END OF THE BOOM WITH THE SHACKLE PROVIDED.

INSERT THE GOOSENECK END IN THE SLOT IN THE MAST, TIE THE SHORT DOQNHAUL ROPE TO THE STAINLESS LOOP AT THE BOTTOM OF THE GOOSENECK FITTING.

- 2 -

RIGGING THE SAILS

“ BENDING ON THE SAILS ”  WITH ELEGANCE IS A SAILOR'S ART WHICH COMES WITH PRACTICE AND EXPERIENCE, DESCRIBED BELOW FOR THE NOVICE IS HOW TO FIT THE RUNNING RIGGING AND RAISE THE SAILS ON YOUR KINGFISHER  III.

THE MAIN SHEET (32') ROPE IS ATTACHED TO THE END OF THE BECKET BLOCK ON THE BOOM AND THEN RUNS THROUGH THE BRASS TRAVELER BLOCK ON THE BACK DECK OF THE HULL., THEN BACK UP THROUGH THE BECKET BLOCK SHEAVE TO THE SWIVEL BLOCK ON THE BOOM (MIDDLE).  THE MAIN SHEET CONTROLS THE POSITION OF THE BOOM AND WHEN SAILING IT IS HELD IN THE HAND OR PASSED THROUGH A SWIVEL JAM CLEAT ON THE FLOORBOARD (OPTIONAL)

THE MAIN SAIL IS UNFOLDED TO LOCATE THE HEAD (TOP). TACK (LOWER FORWARD CORNER) AND THE CLEW (LOWER REAR CORNER).THE 3 BATTENS ARE INSERTED IN THEIR POCKETS ALONG THE LEECH (REAR EDGE). THE MAIN HALYARD IS ATTACHED TO THE HEAD OF THE SAIL.AS THE SAIL IS RAISED BY PULLING ON THE MAIN HALYARD, THE LUFF ROPE IS FED INTO THE SAIL TRACK GROOVE VIA THE SLOT JUST ABOVE THE BOOM. FEED THE ROPE SEWN ALONG THE FOOT (BOTTOM EDGE OF THE SAIL) INTO THE SLOT ON THE BOOM. SECURE THE TACK INTO THE CAPTIVE PIN ON THE TOP OF THE GOOSENECK FITTING. THE OUTHAUL ROPE IS SEWN ON THE CLEW OF THE SAIL. THIS LINE RUNS THROUGHT THE SMALL HOLE IN THE OUTHAUL FITTING, AND BACK ALONG THE BOOM TO THE OUTHAUL CLEAT ON THE SIDE OF THE BOOM. STRETCH THE FOOT OF THE SAIL TIGHT.

THE SAIL SHOULD BE RAISED AS HIGH AS POSSIBLE AND THEN STRETCHED DOWNWARD BY PULLING DOWN ON THE BOOM AND CLEATING THE DOWNHAUL ROPE. SIMILARLY, THE SAIL SHOULD BE STRETCHED OUTWARD BY MEANS OF THE OUTHAUL LINE.

THE TENSION IN BOTH LUFF AND FOOT ARE ADJUSTED FOR WIND AND SAILING CONDITIONS. THE TENSION IN THE LEECH CAN BE ADJUSTED BY THE SPECIAL LEECH LINE PROVIDED.

THE JIB IS UNFOLDED TO LOCATE THE HEAD (TOP) TACK (FRONT LOWER CORNER) AND THE CLEW (REAR LOWER CORNER). THE JIB HALYARD ROPE  (23') IS ATTACHED TO THE HEAD. AS THE JIB IS RAISED THE CLIPS ARE ATTACHED TO THE JIB STAY. THE JIB TACK IS ATTACHED TO THE TACK SWIVEL FITTING. THE JIB HALYARD IS NOW PULLED TIGHT AND ATTACHED TO THE CLEAT ON THE MAST.

THE JIB SHEET (27') IS PASSED THROUGH THE GROMMET IN THE JIB CLEW AND KNOTTED AT THE MIDPOINT. THE ENDS ARE PASSED THROUGH THE STAINLESS FAIRLEAD AND JAM CLEAT ON THE SIDE DECKS. THE JIB SHEET ALWAYS PASSES ON THE OUTSIDE OF THE SHROUDS.

- 3 -

ROLLER REEFING

THE MAINSAIL MAY BE REEFED (SHORTENED) BY ROLLING IT AROUND THE BOOM. THIS FEATURE IS USEFULD IN ROUGH WEATHER OR FOR STORING THE SAIL BETWEEN USES.

    - REMOVE THE BLOCK AT THE MIDDLE OF THE BOOM BY UNSCREWING THE KNURLED SET SCREW AND SLIDING OFF THE WHOLE ASSEMBLY. THE BOOM IS NOW CONTROLLED FROM THE BLOCK AT THE END.

    - LOOSEN THE HALYARD TO RELIEVE TENSION ON THE MAINSAIL

    - PULL OUTWARD (AFT) ON THE BOOM AND ROTATE TO REEF AS DESIRED.

THE RUDDER

THE KICKUP RUDDER IS HELD DOWN BY FRICTION. THE FORCE NECESSARY TO ROTATE OR KICK-UP THE BLADE SHOULD BE CHECKED PERIODICALLY, NECESSARY ADJUSTMENTS MAY BE MADE BY LOOSENING OR TIGHTENING THE PIVOT BOLT. THE RUDDER SHOULD NOT BE USED IN THE UP POSITION SINCE THIS PUTS EXCESSIVE LOADS INTO THE RUDDER AND ASSOCIATED MOUNTING HARDWARE.

THE CENTERBOARD

THE DEPTH OF THE CENTERBOARD IS ADJUSTED BY THE LINE AND V JAM CLEAT ON THE TRUNK SIDE. FOR MOST SAILING ONLY ½ BOARD IS NEEDED. MANY SAILORS MARK THE LINE AT THE ¼, ½ AND ¾ BOARD POSITION WITH A WATERPROOF MARKER PEN.

- 4 -

CARE AND MAINTENANCE OF YOUR KINGFISHER  III

THE KINGFISHER  III HAS BEEN CAREFULLY BUILT OF THE FINEST MATERIALS TO ASSURE YOU OF MANY YEARS OF CAREFREE USE.  CORROSION RESISTANT MATERIALS HAVE BEEN USED THROUGHOUT; INCLUDING MOLDED FIBERGLASS HULL, DECK, TRUNK AND INTERIOR. STAINLESS STEEL RIGGING AND FITTINGS, ANODIZED ALUMINUM MAST AND BOOM. POSITIVE FOAM FLOTATION UNDER THE DECKS.  ALL SAILS AND LINES ARE OF TOP QUALITY SYNTHETIC.

A LITTLE ATTENTION WILL GO A LONG WAY TOWARD MAINTAINING YOUR KINGFISHER'S APPEARANCE AND CONDITION.

THE HULL AND DECK MAY BE CLEANED OF NORMAL DIRT AND STAINS WITH ANY GOOD HOUSEHOLD DETERGENT,  MORE STUBBORN STAINS CAN BE REMOVED WITH A LITTLE CLEANSER SUCH AS AJAX. TAKE CARE NOT TO PENETRATE THE GELCOAT LAYER.

A LIGHT COAT OF LIGHT AUTO TYPE WAX WILL GIVE THE FINISH A HIGH GLOSS AND HELP PROTECT THE FINISH.

IF THE BOAT IS LEFT IN SALT WATER THE BOTTOM SHOULD BE PAINTED WITH AN ANTI-FOULING TYPE OF PAINT. WE RECOMMEND A VINYL OR EPOXY TYPE AND SUGGEST THAT THE MANUFACTURERS DIRECTIONS BE FOLLOWED. THE CENTERBOARD BLADE AND AS MUCH AS POSSIBLE OF THE INSIDE OF THE TRUNCK SHOULD ALSO BE PAINTED WITH THE ANTI-FOULING MATERIAL.

THE SAILS, IF NOT REMOVED COMPLETELY, MAY BE WRAPPED AROUND THE BOOM AND TIED WITH THE SHEET LINE FOR STORAGE. THE SAILS MAY BE HAND WASHED AND AIR DRIED CAREFULLY. IT IS NOT A GOOD POLICY TO LEAVE THE SAILS ON THE BOAT FOR A LONG PERIOD OF TIME.

GOOD LUCK AND GOOD SAILING.

- 5 -